- 龙门丝网焊专机

- Wire Mesh Welding Machine

整机特性：

1、手动送料型： 人工将工件放入模内，启动焊机依次焊接一排，再手工推进到下一排自动焊接，直至焊接完成整个工件；

2、 半自动送料型: 人工摆好工件，自动步进送料，送料到位机器自动焊接一排，再自动拉进到下一排，自动焊接，直至焊接完成整个工件，工件再自动返回或从后方退出；

3、 自动双模送料型:  采用双层式或前后双模式自动焊接装置，利用先进的PLC和人机界面控制及精确的伺服电机驱动系统，组成自动连续的送料和焊接机构，有两套模具，一套在焊接，另一套在摆放工件，两模交替进行，机器可以不间断连续工作，有效的提高了效率；

4、自动送料及自动焊接步进型: 装有自动下料装置，横线放进料斗，纵线自动按距离步进到位 ，然后横线自动落料，自动焊接一排，依次循环，直至需要的整个网片焊完；

广泛应用于：各种场所护栏网、五金网类、货架、笼类、网架类、铁板加强筋等多点式焊接。

Main Features:

- Manual feeding type: Manual place the precut wires into mesh mold, start the welding machine and welding a row in turn, then manual push mold to next row for automatic welding until finish a complete mesh;

- Semiautomatic feeding type: Manual place the precut wires into mesh mold, automatic step feeding to welding position and welding a row automatically, then automatic draw the mesh mold to next welding position and automatic welding until to finish a complete mesh, then mesh mold automatic back to origin and get out the mesh ;

- Automatic Double Mold Feeding Type: Adopted double layers type or tandem type automatic welding fixtures, and combined advanced PLC , human machine interface and precision servo motor driven system to obtain the continuous feeding and welding automatically.

- Machine included double mold for alternating welding, if one performed welding, then another can be placed wires at same time. So the machine can continues working to improve production efficiency;

- Full automatic feeding type: Equipped automatic hopper for cross wire feeding, line wire automatic feeding as setting length through automatic mesh drawing system .

- Widely used for each kinds of fence , hardware mesh, goods shelf, cages, wire frame, steel plate stiffener and others multiple spot welding.