MF系列中频逆变直流点（凸）焊机

MF Series Medium Frequency Inverter DC Spot Welding Machine

整机特性

- 高质量、高效率的焊接能力，功率因数达98%，三相输入负载平衡，对用户电网的冲击大大减少，输出电流为直流，电流平滑，感抗为零，1000HZ的工作频率，时间控制精度为1MS；

- 2、高强度的机体结构，刚性好，垂直式加压机构，导向采用直线导轨，随动性高；

- 3、采用高速微处理器，运用智能最优算化法，液晶屏界面，可存储60套焊接规范，具有中英文切换显示，提供外部BCD码调用接口，系统故障自动诊断报警等功能；

- 4、特别适合焊接导电性较高的铝、铜材料，比较厚的工件。广泛适用于航空、航天、汽车、压缩机及其他领域的高端产品的焊接要求。

Main Features

- High quality,high efficiency,98% power factor,three-phase input load balancing,the impact on the user grid greatly reduced,DC input current,smooth,inductance is zero,1000HZ working frequency,time control precision is 1MS;

- High-strength body structure,good rigidity,direct pressurizing mechanism,linear bearings used in the guiding structure enable good mobility;

- Adopt high-speed  microprocessors,use intelligent optimal calculation method,LCD interface,can save 60sets welding schedule numbers,There are functions of switch display in both Chinese and English,provide external BCD call interface,system fault diagnosis alarm automatically.etc;

- Specially suitable for welding higher conductive rate of material(such as aluminum,copper,silver and so on) and  much more thicker workpiece,widely used in automobile,aerospace and other high request products and industry.

主要技术参数    Main Technical Parameters

| 参数/型号 Parameter/Model | MF-18KA | MF-25KA | MF-32KA | MF-40KA | MF-60KA | MF-80KA |
| --- | --- | --- | --- | --- | --- | --- |
| 额定电源 Rated Voltage | 3相，交流（AC）, 380-----415----480V,50/60HZ. |  |  |  |  |  |
| 额定容量 Rated Capacity | 50KVA | 90KVA | 150KVA | 170KVA | 250KVA | 300KVA |
| 额度占载率 Rated Duty Cycle | 50% | 50% | 50% | 50% | 50% | 50% |
| 最大焊接电流 Max Welding Current | 18000A | 25000A | 32000A | 40000A | 60000A | 80000A |
| 电极行程 Electrode Stroke | 60MM |  |  |  |  |  |
| 电极臂间距 Welding Arm Spacing | 380MM |  |  |  |  |  |
| 气源压力 Air Source Pressure | 0.6MPa |  |  |  |  |  |
| 冷却水流量 Cooling Water Flow | 16L/Minute | 25L/Minute | 35 L/Minute | 45L/Minute |  |  |